CENG 4130 — Plant Design and Economics
Final Examination, Paper B (Spring 2025/2026)
Worked Solutions

Student ID: 20987955    |    Development cost X = USD 20,987,955

# Question 1 — VacuumBot Project Economics

## 1.0  Basis and assumptions

Year 0 captures all pre-operational outlays (1-year development period plus equipment purchase and working capital injection at start-up). Operations and sales span Years 1–5.

Annual sales volume, selling price, fixed and variable operating costs, and advertising budget are constant over the 5-year operating life.

Depreciation: 5-year property class MACRS half-year convention with rates 20.00 / 32.00 / 19.20 / 11.52 / 11.52 / 5.76 %. The project is terminated at the end of Year 5, so the un-recovered 5.76 % of the equipment basis (USD 2,304,000) is treated as a book loss on disposal (salvage = 0) and generates a one-off tax shield in Year 5.

Development cost X is treated as a non-depreciable, non-deductible pre-operating outlay (capitalised). Treating it as a Year-0 tax-deductible expense would only improve NPV and IRR; the assumption taken is conservative.

Profits tax rate = 15 % (Hong Kong two-tier upper rate). Discount rate = 20 % per annum, end-of-year cash-flow convention.

Working capital of USD 20 M is fully recovered at the end of Year 5.

## 1.1  Annual revenue and operating cost

| Item | Calculation | Value (USD/yr) |
| --- | --- | --- |
| Revenue | 500,000 units × USD 250 / unit | 125,000,000 |
| Variable cost | 500,000 × USD 90 | 45,000,000 |
| Fixed operating cost | given | 35,000,000 |
| Advertising | given | 8,000,000 |
| Total operating cost | 45 + 35 + 8 | 88,000,000 |
| EBITDA (Revenue – OpCost) | 125 – 88 | 37,000,000 |

## 1.2  Depreciation schedule (5-year MACRS on USD 40 M)

| Year | MACRS rate | Depreciation (USD) | Book value EoY (USD) |
| --- | --- | --- | --- |
| 1 | 20.00 % | 8,000,000 | 32,000,000 |
| 2 | 32.00 % | 12,800,000 | 19,200,000 |
| 3 | 19.20 % | 7,680,000 | 11,520,000 |
| 4 | 11.52 % | 4,608,000 | 6,912,000 |
| 5 | 11.52 % | 4,608,000 | 2,304,000 |

Un-recovered book value of USD 2,304,000 at end of Year 5 is written off as a loss on disposal; tax shield = 0.15 × 2,304,000 = USD 345,600.

## 1.3  After-tax cash-flow table

| Year | Revenue | OpCost | Depr. | EBT | Tax (15 %) | Net CF |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | — | — | — | — | — | (80,987,955) |
| 1 | 125,000,000 | 88,000,000 | 8,000,000 | 29,000,000 | 4,350,000 | 32,650,000 |
| 2 | 125,000,000 | 88,000,000 | 12,800,000 | 24,200,000 | 3,630,000 | 33,370,000 |
| 3 | 125,000,000 | 88,000,000 | 7,680,000 | 29,320,000 | 4,398,000 | 32,602,000 |
| 4 | 125,000,000 | 88,000,000 | 4,608,000 | 32,392,000 | 4,858,800 | 32,141,200 |
| 5 | 125,000,000 | 88,000,000 | 4,608,000 | 32,392,000 | 4,858,800 | 52,486,800 (+WC 20.0 M + tax shield 0.346 M) |

Year-0 outlay = X (20,987,955) + Equipment (40,000,000) + Working capital (20,000,000) = USD 80,987,955.

## 1.4  (a) Project cash position

| Year | Annual cash flow (USD) | Cumulative cash position (USD) |
| --- | --- | --- |
| 0 | (80,987,955) | (80,987,955) |
| 1 | 32,650,000 | (48,337,955) |
| 2 | 33,370,000 | (14,967,955) |
| 3 | 32,602,000 | 17,634,045 |
| 4 | 32,141,200 | 49,775,245 |
| 5 | 52,486,800 | 102,262,045 |

The cumulative cash position turns positive between Year 2 and Year 3. Undiscounted simple payback period = 2.46 years.

Figure 1. Cumulative project cash position (Year 0 to Year 5).

## 1.5  (b) Net present worth at 20 %

Discounting each annual cash flow at i = 20 %:

| Year | Cash flow (USD) | Discount factor (1.20)^-n | Present value (USD) |
| --- | --- | --- | --- |
| 0 | (80,987,955) | 1.0000 | (80,987,955) |
| 1 | 32,650,000 | 0.8333 | 27,208,333 |
| 2 | 33,370,000 | 0.6944 | 23,173,611 |
| 3 | 32,602,000 | 0.5787 | 18,866,898 |
| 4 | 32,141,200 | 0.4823 | 15,500,193 |
| 5 | 52,486,800 | 0.4019 | 21,093,268 |
| Σ NPV |  |  | 24,854,348 |

NPV @ 20 % = +USD 24,854,348 (≈ +USD 24.9 million).  Because NPV > 0 at the firm’s hurdle rate, the project creates value and should be undertaken.

## 1.6  (c) Discounted cash-flow rate of return (DCFROR)

Solving Σ CFₙ / (1+i)ⁿ = 0 by iteration (bisection on i):

| Trial discount rate | NPV (USD) |
| --- | --- |
| 20.0 % | 24,854,348 |
| 25.0 % | 13,544,979 |
| 30.0 % | 4,102,067 |
| 32.5 % | (43,916) |
| 35.0 % | (3,859,971) |

DCFROR ≈ 32.47%.  Since the DCFROR substantially exceeds the 20 % hurdle, the project is economically attractive.

## 1.7  Robustness checks (subtleties verified)

MACRS disposal accounting: Textbook convention used: Y1–Y5 take full rates 20/32/19.20/11.52/11.52 % (Σ = 94.24 %) and the un-recovered 5.76 % becomes a disposal loss in Y5. Under the strict IRS half-year-of-disposal rule, Y5 depreciation would instead be 5.76 % and the disposal loss 11.52 %. In both treatments the Y5 total deductible amount is identical (5.76 % + 5.76 % = 11.52 % = USD 4.608 M), so Y5 tax = USD 4.514 M and Y5 cash flow = USD 52.487 M either way. NPV and IRR are therefore invariant to which MACRS-disposal convention is used.

Tax treatment of the development cost X: I capitalised X as a non-depreciable, non-deductible pre-operating outlay (conservative). If instead X is expensed in Year 0 against parent-company taxable income, an immediate tax shield of 0.15 × X = USD 3,148,193 is added to Y0. NPV(20 %) then rises to ≈ USD 28.0 M and DCFROR to ≈ 34.5 %.

Timing convention for pre-operating cash outlays: I lumped X, equipment and working capital at Year 0 (operations Y1–Y5) — the standard textbook treatment used in most plant-economics courses. Two alternatives were tested: (i) A pure 1-year time-shift of the whole cash-flow stream (entire profile pushed back by the development year). Mathematically this divides NPV by (1+r) but leaves the IRR root unchanged: NPV(20 %) ≈ USD 20.7 M, DCFROR ≈ 32.5 %. (ii) Separating the outlays — X at Y0, equipment + WC (USD 60 M) at Y1, operating cash flows at Y2–Y6 — which postpones the bulk of capex by one year but also postpones revenue: NPV(20 %) ≈ USD 17.2 M, DCFROR ≈ 28.9 %. In every case NPV is positive and DCFROR exceeds the 20 % hurdle, so the accept-the-project conclusion is robust to the timing assumption.

Production = sales: Variable cost is given per unit "produced". Assumed no inventory build-up, so production = sales = 500,000 units / yr.

Advertising deductibility: Treated as an annual tax-deductible operating expense (standard for HK profits tax).

Working capital: Single injection at start-up, fully recovered at end of Year 5 (no tax effect on recovery).

Discount-rate / convention: End-of-year cash flow convention at i = 20 % p.a., consistent with the discrete annual cash-flow table.

## 1.8  Summary of Q1 results

| Metric | Value |
| --- | --- |
| Total Year-0 outlay | USD 80,987,955 |
| Annual EBITDA (Years 1–5) | USD 37,000,000 |
| Simple payback (undiscounted) | 2.46 yr |
| NPV at 20 % | +USD 24,854,348 |
| DCFROR (IRR) | 32.47 % |
| Recommendation | Proceed with the project |

# Question 2 — Towngas Residential Hydrogen Refilling Facility

Role: Technical Lead, Towngas — H₂ extraction by PSA from existing town-gas mains, installed in a residential car-park bay (≈3 parking spaces ≈ 37.5 m² footprint).

## Part (a) — Economic Feasibility Evaluation (Board of Directors)

### A1. Process scope and design basis

Town gas composition (Towngas, naphtha-reformed): ≈49 vol% H₂, 28 % CH₄, 19 % CO₂, 3 % CO, balance N₂. Distribution pressure ≈ 75 mbarg, taken locally up to ≈ 8–10 barg by a feed booster.

PSA recovery 80 %, H₂ purity 99.97 % (SAE J2719 fuel-cell grade).

Nominal H₂ production: 50 kg H₂ / day. Storage at 875 bar (Type IV vessels) with cascade dispensing at 700 bar (H70).

Site footprint = 37.5 m² (3 parking bays); 24/7 operation; expected utilisation ramp 40 % (yr 1) → 75 % (yr ≥3); design life 15 yr.

Refuelling spec: 4–5 kg H₂ filled in <5 min per FCEV (Toyota Mirai ≈ 5.6 kg → ~650 km).

Assumed FCEV throughput: ~7 vehicles / day at design utilisation; plus 30 kg/day diverted to a stationary 50 kWₑ PEM fuel-cell unit feeding the building.

### A2. Capital cost estimate (Class-4 study estimate, ±30 %)

| Item | Basis | Installed cost (USD) |
| --- | --- | --- |
| Town-gas feed booster + dryer + pretreatment | 10 Nm³/h skid | 60,000 |
| PSA unit (4-bed) sized for 50 kg H₂/day | small-scale skid pricing | 450,000 |
| Multi-stage diaphragm compressor 0.5–900 bar | ~30 kg/h, 50 kW | 380,000 |
| High-pressure storage cascade (≈100 kg @ 950 bar) | Type IV composite | 260,000 |
| H₂ dispenser (H70) + chiller (–40 °C) | single-hose | 180,000 |
| Civil works, fire wall, vent stack, ventilation | small footprint | 120,000 |
| Instrumentation, SIS, gas detection, CCTV | SIL-2 safety system | 110,000 |
| Engineering, procurement, commissioning, licensing | 15 % of direct | 230,000 |
| Contingency | 15 % of installed | 270,000 |
| Total fixed capital investment (FCI) |  | ≈ 2,060,000 |

Working capital ≈ 10 % FCI = USD 0.21 M. Total capital outlay ≈ USD 2.27 M per site.

### A3. Annual operating cost (at 75 % utilisation, 13,700 kg H₂/yr)

| Cost element | Basis / unit cost | USD / yr |
| --- | --- | --- |
| Town-gas feed (net H₂ extracted only) | ≈ 0.5 USD / kg H₂ (HK Towngas wholesale ≈ HK$1.2/m³; depleted stream returned to grid at credit) | 6,850 |
| Electricity (PSA + compression + chiller ≈ 9 kWh/kg) | 13,700 kg × 9 × USD 0.16 / kWh | 19,700 |
| Cooling water + N₂ purge | — | 3,000 |
| Maintenance + spares | 4 % FCI | 82,000 |
| Labour (0.3 FTE shared across cluster) | 0.3 × USD 90 k | 27,000 |
| Insurance & permits | 1.5 % FCI | 31,000 |
| Property rental (3 bays) | 3 × USD 350/mo × 12 | 12,600 |
| Total OPEX (excl. depreciation) |  | ≈ 182,000 |

### A4. Levelised cost of hydrogen (LCOH)

Annualisation factor (capital recovery factor) at 8 % over 15 yr: CRF = i(1+i)ⁿ / [(1+i)ⁿ – 1] = 0.1168.

Annualised capex = 2,060,000 × 0.1168 = USD 240,600 / yr.

LCOH = (annualised capex + OPEX) / annual H₂ delivered = (240,600 + 182,000) / 13,700 ≈ USD 30.8 / kg H₂.

| Utilisation | Annual kg H₂ | LCOH (USD/kg) | Equiv. per 100 km* |
| --- | --- | --- | --- |
| 40 % | 7,300 | 56.0 | ≈ USD 56.0 |
| 60 % | 10,950 | 38.1 | ≈ USD 38.1 |
| 75 % | 13,700 | 30.9 | ≈ USD 30.9 |
| 90 % | 16,425 | 26.1 | ≈ USD 26.1 |

*Toyota Mirai consumption ≈ 0.86 kg H₂ / 100 km; rounded to 1 kg / 100 km. Gasoline-ICE equivalent: 7 L/100 km × HK$ 25 /L (~USD 3.2 /L) ≈ USD 22 / 100 km.

### A5. Parity with gasoline on the three stated criteria

| Criterion | Gasoline reference | This H₂ facility |
| --- | --- | --- |
| Cost per 100 km of service | 7 L gasoline × HK$25/L ≈ USD 22 / 100 km | ≈ 1 kg H₂ × LCOH = USD 18–31 / 100 km (USD 14–18 with cluster procurement) |
| Refuelling time (5 kg fill) | ≈ 3–5 min | ≈ 3–5 min via H70 fast-fill (–40 °C pre-cooled cascade), bounded by SAE J2601 protocol |
| User convenience | Self-service nozzle, card payment, ~24 / 7 access | Self-service H70 nozzle with break-away coupling, RFID + Octopus / app payment, geofenced 24/7 access, identical UX |

### A6. Benchmark and conclusion

At design utilisation, distributed H₂ from this PSA route delivers H₂ at ≈ USD 18–31 / kg depending on utilisation. Gasoline equivalent on energy-service basis is ≈ USD 22 / 100 km, hence H₂ becomes cost-competitive only above ≈70 % utilisation and after factoring HK ZEV incentives and the 50 % first-registration-tax waiver on FCEV.

A residential cluster roll-out of ≈ 50 sites yields economy of scale on PSA, compressor and dispenser procurement (≈25 % FCI reduction) and shared maintenance crew, dropping LCOH to ≈ USD 14–18 / kg — broadly comparable with gasoline on a per-km basis.

Scalability: town-gas backbone covers >90 % of HK residential premises; no new feedstock distribution is required, making per-site scale-up faster than electrolysis (which is electricity-grid limited).

Major economic risks: (i) low FCEV fleet uptake → low utilisation; (ii) town-gas tariff escalation; (iii) carbon-intensity scrutiny — Towngas is fossil-derived, so CO₂ from the depleted return stream still counts against the building portfolio unless CCS is added downstream; (iv) regulatory: HK Electrical & Mechanical Services Department (EMSD) licensing for residential H₂ is still maturing; (v) safety incident → loss of social licence.

Sensitivity (per-site, design utilisation): every +10 % utilisation reduces LCOH by ≈ USD 2.6 / kg; every USD 0.02 / kWh increase in electricity adds ≈ USD 0.18 / kg.

Recommendation: Pilot a single residential site for 12 months. Approve full roll-out only if (a) utilisation > 60 % by end of pilot, (b) safety case approved by Fire Services Department and EMSD, and (c) cluster procurement secures ≥20 % capex reduction.

## Part (b) — Safety and Risk Management Proposal (Government Submission)

### B1. Key hazards identified

Hydrogen flammability: very wide flammable range 4–75 vol% in air, minimum ignition energy ~0.02 mJ (≈10× lower than methane). Quasi-invisible flame (≈ 200–300 °C surface but very high radiant heat).

High-pressure mechanical hazard: 700–950 bar storage; potential for catastrophic vessel rupture, jet release (sonic, > 1,300 m/s), jet fire.

Confined space accumulation: car-park is partially enclosed → loss of containment could pool above the leak and form a flammable cloud near the soffit (H₂ rises at ~20 m/s buoyancy in air).

Hydrogen embrittlement of carbon-steel piping and fittings; cyclic-fatigue failure of refuelling hose.

Town-gas side hazards: methane / CO toxicity of the feed; backflow of H₂ into low-pressure mains.

Electrical ignition sources (vehicles, lighting), static, lightning.

Human interaction: untrained residents performing self-service refuelling; vehicle impact on dispenser; vandalism.

Domino effect with adjacent parked EVs / ICE cars (Li-ion thermal runaway, gasoline fuel tank).

### B2. Qualitative risk assessment (HAZID / What-If summary)

| Scenario | Cause | Consequence | Likelihood | Risk (pre-mitigation) |
| --- | --- | --- | --- | --- |
| Small H₂ leak at compressor seal | Seal wear | Local ignition, jet fire <2 m | Possible | Medium |
| PSA tail-gas H₂ slip → back-feed town-gas line | PSA control failure | Pipeline over-pressure, off-spec gas to consumers | Unlikely | Medium |
| Dispenser hose burst during refuelling | Embrittlement / impact | Jet flame, burn injury, vehicle fire | Possible | High |
| Storage-tank BLEVE / rupture | Fire engulfment / overfill | Blast over-pressure, fragments, fatalities | Rare | High |
| Flammable cloud in enclosed car-park | Sustained leak + poor ventilation | Deflagration / DDT, structural damage | Unlikely | High |
| Town-gas (CH₄/CO) feed leak | Pipework failure | Toxic exposure, asphyxiation | Possible | Medium |
| Vehicle collision with dispenser | Driver error | Mechanical damage, possible leak | Possible | Medium |
| Loss of power → SIS de-energised | Grid failure | Process upset, isolation valves close (fail-safe) | Possible | Low |

### B3. Engineering & administrative safeguards

Layout: outdoor or semi-outdoor canopy with at least two open sides; minimum separation distances per NFPA 2 (2023) — ≥ 6 m from public-access edge, ≥ 8 m from building openings, ≥ 3 m from parked vehicles. Fire wall (REI-120) between PSA / storage skid and adjacent parking bays.

Ventilation: natural cross-ventilation ≥ 1 % of canopy roof area; supplementary forced extraction at high level (≥ 12 air changes per hour) to prevent H₂ accumulation > 25 % LFL (=1 vol%).

Detection: redundant H₂ point detectors (catalytic + thermal-conductivity) at the soffit, ultrasonic leak detectors near compressors, UV/IR flame detectors at storage. Two-out-of-three voting triggers trip at 20 % LFL.

Safety Instrumented System (SIL-2): fail-safe isolation valves on town-gas inlet, H₂ outlet, storage and dispenser; automatic depressurisation to a remote vent stack (≥ 4 m above roof) on confirmed leak/fire.

Mechanical integrity: all wetted parts in 316L SS (low-strength grade to resist embrittlement); double-block-and-bleed on the town-gas / H₂ interface; non-return valves to prevent back-feed; hoses replaced on time-in-service or cycle count.

Hazardous-area classification (ATEX / IEC 60079): Zone 1 within 1 m of all H₂ joints, Zone 2 within 3 m; explosion-proof Ex-d luminaires, intrinsically-safe instruments; bonding and earthing of all metallic parts; lightning protection.

Pressure relief: thermally-activated PRDs on every storage cylinder, vented through dedicated vent stack with flashback arrestor; PSV on PSA receivers sized for credible heat-input case.

Vehicle protection: bollards (kerb-mounted, K-12 rated) around dispenser and storage; CCTV; dispenser nozzle with automatic break-away coupling.

Operational: read-only HMI for residents; QR-coded ID + RFID authorisation; geofenced shut-off if smoking / open flame detected; mandatory training for housing-estate maintenance staff; permit-to-work for hot-work / breaking containment.

Documentation: site-specific HAZOP + LOPA, Pre-Startup Safety Review (PSSR), Management of Change (MOC), incident reporting under EMSD Gas Safety Ordinance Cap. 51.

### B4. Credible worst-case scenario and emergency response

Credible worst case (CWC): catastrophic failure of one 95 L composite cylinder at 950 bar full of H₂ (≈ 6 kg H₂, ≈ 0.85 GJ). Outcomes envelope:

Immediate ignited release → vertical jet fire ~6–8 m flame length; thermal radiation 12.5 kW/m² endpoint at ≈ 6 m → exclusion zone radius 8 m maintained by layout.

Delayed ignition with confined accumulation → deflagration peak overpressure ≈ 0.3 bar at 5 m, dropping below 0.05 bar at 15 m (structural-damage threshold). Achieved mitigation through ventilation and explosion-relief louvers in canopy.

Unignited release → buoyant dispersion above roof line; ≤ 30 s to reach upper LFL contour, well below combustion-air domain at ground level.

Mitigation hierarchy applied: inherent (small inventory per cylinder, 100 kg total cap), passive (composite-wrapped vessels with TPRD), active (SIS isolation in < 1 s, blow-down to vent stack within 90 s), procedural (training, drills).

### B5. Emergency Response Plan (ERP) — key elements

Tier-1 (operator): automatic SIS trip — isolation + venting + sirens + strobe; SMS / app alert to estate manager and Towngas control room.

Tier-2 (on-site): evacuation of car-park within 60 s using lit egress paths; activation of fire-suppression for ignited release (water curtain to cool adjacent surfaces — do NOT extinguish a controlled jet fire before isolation is confirmed).

Tier-3 (external): notification to Fire Services Department (FSD) and EMSD; pre-agreed access route for fire appliances; ≥ 100 m cordon for ignited storage event.

Post-incident: lock-out / tag-out, third-party root-cause analysis, regulator submission within 24 h per Gas Safety (Installation and Use) Regulations.

Drills: full-scale exercise with FSD annually; quarterly tabletop with estate management; pre-commissioning and re-validation every 5 yr.

Public communication: pre-launch information to residents — what hydrogen is, how to recognise an alarm, evacuation route, no smoking radius. Posters in 中/EN at lift lobbies and car-park entrances.

### B6. Overall safety conclusion

With inherent design choices (small distributed H₂ inventory, semi-outdoor location, PSA/compressor separation, composite vessels with TPRDs), engineered safeguards (SIL-2 SIS, detection, ventilation, vent stack), and a structured ERP integrated with FSD and EMSD, the residual risk of fatality at the boundary of the car-park can be demonstrated to be Tolerable / ALARP (≤ 1×10⁻⁶ per year individual risk). The proposal is judged acceptable for pilot deployment subject to formal HAZOP, LOPA and Quantitative Risk Assessment (QRA) submission to the regulator prior to commissioning.